Queries – 27-09-2026

| ID | Decision | Information required | Response from Scott |
| --- | --- | --- | --- |
| D01 | Use environment | Land yacht/desert versus marine/sailboat description; water, dust, cleaning, vibration, impact and thermal conditions. We assume you'd like the enclosure and interfaces to be able to withstand marine water and desert dust while mated, but not whilst no connector is fitted. Is that correct or do you require the connectors to be waterproof and dustproof when unmated? Are there any vibration requirements you have? (i.e. is the land sailboat subjected to a lot of vibration due to the desert racetrack?) | Def focused on land use (although water proof would be a plus). We don’t need dust proof when unmated. Moreover, be fine to have one cover on power/usb ports. Def vibration – similar to car on dirt road. The playa we race on is like concrete though – so pretty smooth. |
| D02 | Actual prototype baseline | Can you provide further information on the exact model numbers of Pi Zero 2 W and Pi 4, exact GNSS board and revisions, telemetry and correction-radio SKUs, current wiring and interfaces? Photos of the model numbers printed on each board would help, just to make sure that we're considering exactly the same revision of hardware. | I want to use “Pi Zero 2”s, but they are impossible to find. The old Pi Zeros do work – have tested, but the 2 has so much more horse power. The Pi 3 B+ is what I’m testing with in field, but it’s the last model rev and has slightly larger foot print. Ideal world we get “Zero 2”s, and maybe bridge with the old “Zero”. I’ll attach a wiring diagram with part numbers. |
| D03 | Course distances and acceptance | In our emails we were talking about distance for the race. You mentioned 10 km would probably be a worst case. Please can you confirm if a 10 km race would be with a base receiver located at 5 km (so at the midway point) or if the base unit needs to be located at the beginning or end of the race? In other words, define whether 2 km, 5 km and 10 km refer to course span or one-way radio path. Also define packet delivery and maximum update gap at range. | The playa is 2mi or 3.2 km. The RTK sending the RTCM messages would be max 2mi away from boats. The telemetry receiver will be in the center of the course so 1mi (1.6km). Keep in mind the finish is the most important it will be 100m from the telemetry receiver. We are shooting for 5hz close to the finish (100m) and 2hz everywhere else. Testing showed telemetry range of at least 2mi with antenna at 5.4m above playa. |
| D04 | Finish and mark performance | Required position accuracy, time error and acceptable live latency at start, marks and finish; number of simultaneous finishers. | Love to see the 1cm accuracy in the finish zone. Latency not a big concern, if we are sub 3sec we’d be fine. 5 max finishers simultaneous |
| D05 | GNSS rate | Actual configured NAV-PVT rate, constellation mode, logging rate, receiver capability and whether 10 Hz live updates are genuinely needed. | I think 5Hz would be fine. Right now the rover is set to 10Hz, and the rover has a distance trigger of 1m right now (adjustable). Assuming 30mph 10Hz 1.3m between sends 5Hz 2.6m. Boat length is 5m so either be pretty good. |
| D06 | Correction service | Exact RTCM radio at both ends, band, traffic rate, correction age and RTK fixed/float behaviour with telemetry transmitting. | Best source here is I’m using the stock ArduSimpleRTK boards. I’ll attach all the data and config I have. |
| D07 | Radio profile conflict | Integration reference says TO = 0xC0 DigiMesh and P2MP is incompatible; later data dump shows TO = 0x40. Please identify working end-to-end firmware and mode. | Docs issue as I switched to P2MP. I’ll dump the latest configs in the docs attached. |
| D08 | Protocol change | Once you have your results from the trial, are you planning to change 27-byte legacy frames with multiple GPS positions per message (e.g. 4 GPS positions)? | Batch of 4 fixes/frame is the optimal as per my last email. Field testing verifies it holds up under distance. |
| D09 | Congestion evidence | Provide raw test logs, number and spacing of physical transmitters, packet delivery/gap metrics and the meaning of sync errors; the two supplied runs have different error figures. How far apart are the boats during the race from a parallel ‘race lane’ viewpoint (i.e. do they start off 10 m apart on the start line for example)? | I’ll send the logs – I only had one test generator in the field with me though – so it’s not going to be that good of a congestion test. |
| D10 | Power target | Battery runtime required, charging, what is the 12-14.4 V source (i.e. is this a 12V SLA battery fitted to a boat?. Can you clarify the whole-unit current by state and the measurement point/voltage/duty cycle for the reported 280 mW radio figure? | No power system on boat. Unit will need to be self powered 12 hour run time minimum, 36 would be great. Assuming 12 hour daily runtime with auto on/off at night. I basically don’t want to rely on people turning the equipment off/on. |
| D11 | Wake behaviour | Decide scheduled versus RF-triggered wake, permitted boot time and whether any radio or GNSS rail must remain powered in standby. For example when the base module transmits a wake signal, all the devices could listen around once a minute, and go to sleep again in between. When an RF carrier is detected they could then remain awake to start the race. This could be a good option without any user intervention. We could then have a timeout period where, if no RF activity happens for, say, 10 minutes, the device will go back to sleep again and periodically listen for the wake signal again. Would be keen to get your thoughts on this so we can understand the size of battery we need to consider during the detailed design phase, helping us determine estimated dimensions for the report. | Hmmm – I like the RF wake – would def work. Right now we are broadcasting the course gps positions every 60s and could easily use that or happy to implement a simpler ping/wake on the base side. Base is hooked up to constant power source. My initial thought was just a simple timer – turn off at night. But this would be tons more efficient. Side note – we have multiple fleets that don’t race at the same time and turning on a subset of the rovers would also be nice. |
| D12 | Physical envelope | Maximum external dimensions will be determined out of this report once we've gathered all the data but are there any limitations regarding mass, mounting geometry, antenna positions, connectors, service access and initial IP target? If we're covering both water and dust ingress protection, I would suggest IP67 to cover both scenarios. | Small and aero dynamic. Plan is to put them in front of the mast step – I’ll include a diagram. |
| D13 | Build quantities | Confirm engineering prototype count and the basis for a 100-unit production price; the early brief says 1-5 prototypes. I would suggest we produce 12 initial prototypes, 2 to be held back by Innovisory for debugging and testing, in parallel with you doing testing with 10 devices. By having 10 devices this should give you enough simultaneous transmission for an RF collision domain test. | Agreed |
| D14 | Software interface ownership | Provide the client-owned README, wiring, runtime configuration and software interface specification; name an owner to confirm that any hardware-driven software change is separately scoped. |  |